APPENDIX C

BANK OBJECTIVES AND PERFORMANCE STANDARDS

APPENDIX C.1:

C.1.1.	Requirements for Bank Objectives and Performance Standards

A.  Implementation of the Columbia River Wetland Mitigation Bank is anticipated to result in substantial gains in aquatic ecosystem functions, as compared to those now present, or those that would likely accrue on the site if the Bank were not constructed.  The Sponsor must be able to demonstrate tangible aquatic ecosystem gains before Bank credits can be awarded for sale, use, or other transfer, because these functional gains will be used to offset comparable losses to other components of the aquatic environment in the Bank service area.  The Bank’s success will be measured by the enumerated objectives, each of which is subdivided into specific performance standards.  The prescribed performance standards each provide a gauge for measuring the success of the ecological restoration and enhancement efforts at the Bank.

B.  Unless otherwise noted, all documentation required for demonstrating attainment of performance standards will be submitted to the IRT for review and approval as a condition of credit award.  Documentation can typically be included in required monitoring reports.  IRT award of credits will be reflected in a letter issued using a joint letterhead and signed by the Co-Chairs.

C.  Recreational, educational, and scientific activities that do not conflict with the use limitations or other provisions of the conservation easement do not interfere with the delineated purposes and goals of the Bank, and do not adversely affect the ecological viability and functionality of the Bank may take place on the Bank site.  As an obligation independent of this Instrument the City of Vancouver requires that activities on the bank site also comply with the Vancouver Municipal Code including, but not limited to, VMC 20.740, Critical Areas Protection, VMC 20.450, Open Space Districts, and 20.760, Shoreline Management Areas.

C.1.2  	Bank Objectives and Performance Standards

Objective 1:  Protect Aquatic Ecosystem Functions

Permanently protect aquatic ecosystem functions at the Bank by instituting the Instrument and implementing a conservation easement with permanent funding for site stewardship.  Each of the performance standards associated with this objective must be met before any Bank credits may be awarded and before any construction or other implementation activities may be initiated pursuant to this Instrument.  Any construction or implementation activities conducted on-site prior to the inception of the establishment period must cease as of the effective date of this Instrument pursuant to Article VI.B.1, until the Objective 1 performance standards have been accomplished.  The initial award of credits in recognition of accomplishment of these performance standards will serve as the IRT’s notification that construction and implementation activities are authorized to commence.

| Performance Standard | Documentation |
| --- | --- |
| 1A) Complete the development of an appropriate Mitigation Banking Instrument and Appendix. | Mitigation Banking Instrument has been signed by the Sponsor and the applicable regulatory agencies. An original signed Instrument must be provided to each of the signatories. |
| 1B) Protect ecosystem function by placing an IRT-approved conservation easement on the property. | Provide the IRT a copy of the signed, IRT-approved conservation easement and evidence that it has been recorded with the Clark County Auditor and placed on the property title. |
| 1C) Provide financial assurance by establishing an IRT-accepted financial assurance mechanism pursuant to the requirements established in Article III.C.1. of the Instrument. | Demonstrate to the IRT that a compliant and acceptable financial assurance mechanism has been established to provide financial assurance for the establishment period. |
| 1D) Establish and fund a Long-Term Management and Maintenance Endowment Fund escrow account, and develop an escrow agreement, all pursuant to the requirements established in Article III.C2. of the Instrument. | Demonstrate to the IRT that a Long-Term Management and Maintenance Endowment Fund has been initiated through establishment of a compliant and acceptable escrow account. Enter into an escrow agreement with the Corps and Ecology. |

Objective 2:  Hydrology

Create wetland hydrology in the excavated portions of the bank site.  Maintain existing wetland hydrology on remainder of the site.

| Performance Standard | Documentation |
| --- | --- |
| 2A) Grading completed according to IRT approved plans. | As-built drawings showing completed grading are approved by the IRT. |
| 2B) A minimum of 108 acres of the site will have wetland hydrology present at 3 years following approval of As-built grading plan. | To demonstrate wetland hydrology, soil will be saturated to the surface, or there will be free water in soil pits at 12 inches or less below the soil surface for at least 10% of the growing season, where the growing season is defined as March 1 through October 31. A monitoring report showing data from wells and soil pits sufficient to document the extent of wetland hydrology on site is approved by the IRT. |
| 2C) A minimum of 108 acres of wetland will be present on the site at 5 and 10 years following approval of As-built grading plan. | The wetlands on site will be delineated according to the 1987 Corps of Engineers Delineation Manual and appropriate supplements as well as the current Washington State wetland delineation manual in effect at the time of delineation. Wetland delineation report is approved by the IRT. |

Objective 3:  Vegetation

Establish a mosaic of native wetland communities, with high interspersion of wetland and upland habitat types on site.

Note: “Cover” is used in this MBI to mean the actual proportion of the ground surface of the sample plot that is covered by a vertical projection of foliage (by single species or defined group of species) as viewed from above (or below for taller shrubs and trees), or by bare substrate.

Performance Standards for All Areas of the Site:

| Performance Standard | Documentation |
| --- | --- |
| 3A) Planting of site completed according to IRT approved plans. Provide a modified financial assurance that conforms to the required amount for a letter of Credit, or the required penal sum for a Surety Bond, as specified in Article III.C.1.d.(i) of the Instrument. | As-built planting plan showing completed planting approved by the IRT. Include a species list, plant spacing and density, and final planted acreages of vegetative community types. Portions of the BPA easement are included in planting area. Demonstrate to the IRT that a compliant and acceptable modified financial assurance has been established. |
| 3B) Within each habitat type (PAB, PEM, PSS, PFO, AFP) including where those habitat types occur under the BPA easement, Himalayan blackberry, tansy ragwort, Canada thistle, and bull thistle do not collectively exceed 15% cover at 3, 5, 7, and 10 years following approval of As-built planting plan. Cover is not averaged for the entire site – maximum applies to each habitat type. Additional species may be added to this list based on site conditions, as negotiated between the Sponsor and IRT. | Monitoring reports documenting non-native invasive species presence and percent cover approved by IRT. Document the percent cover of invasives in each data plot at years 3, 5, 7, and 10. |
| 3C) Within each habitat type (PAB, PEM, PSS, PFO, AFP,) including where those habitat types occur under the BPA easement, cover of reed canary grass and meadow foxtail does not collectively exceed 30% at 3, 5, 7, and 10 years following approval of As-built planting plan. Cover is not averaged for the entire site – maximum applies to each habitat type. | Monitoring reports documenting non-native invasive species presence and percent cover approved by IRT. Document the percent cover of invasives in each data plot at years 3, 5, 7 and 10. |
| 3D) Over the entire site, including the BPA easement, zero tolerance of Japanese knotweed (and related hybrids), purple loosestrife, and English ivy colonization is maintained. Map any specimens and eradicate during growing season of same year. Additional species may be added to this list based on site conditions, as negotiated between the Sponsor and IRT. | Monitoring reports documenting identification and eradication approved by the IRT. Inventory annually and include in monitoring reports at years 3, 5, 7, and 10. |

Performance Standards for Palustrine Aquatic Bed Wetland:

| Performance Standard | Documentation |
| --- | --- |
| 3E) Palustrine Aquatic Bed areas show an increase in species richness over baseline conditions (baseline data is collected during summer of 2010) at 3, 5, 7, and 10 years following approval of As-built planting plan. | Monitoring reports documenting data from random sampling points and waterfowl exclosures approved by IRT. Record species present within plots and the percent cover of each species. Document baseline conditions in 2010 and change over baseline in years 3, 5, 7, and 10. Provide photos from established photo points. |

Performance Standards for Palustrine Emergent Wetland:

| Performance Standard | Documentation |
| --- | --- |
| 3F) In the Palustrine Emergent Wetland, including those PEM areas within the BPA easement, native emergent plant species have a minimum of 40% cover at 1 year, 50% cover at 3 years, and 60% cover at 5, 7, and 10 years following approval of As-built planting plan. | Monitoring reports documenting native species presence and percent cover approved by IRT. Document baseline condition in 2010 and change over baseline emergent cover in years 1, 3, 5, 7 and 10. |
| 3G) The moisture index, as defined in Appendix F, within the Palustrine Emergent Wetland is less than 3.0 | Monitoring reports documenting species occurrence and moisture index approved by the IRT. Document the moisture index in each data plot within the emergent area at years 1, 3, 5, 7 and 10. |

Performance Standards for Palustrine Scrub-shrub Wetland:

| Performance Standard | Documentation |
| --- | --- |
| 3H) In the Palustrine Scrub-shrub Wetland, native woody species have a minimum of 20% cover at 3 years, 40% cover at 5 years, 50% cover at 7 years, and 60% cover at 10 years following approval of the As-built planting plan. | Monitoring reports documenting species occurrence approved by the IRT. Document the percent cover of native woody vegetation in years 3, 5, 7, and 10. |
| 3I) The moisture index in the Palustrine Scrub-shrub Wetland is equal to or less than 3.0. | Monitoring reports documenting species occurrence and moisture index approved by the IRT. Document moisture index for years 3, 5, 7, and 10. |

Performance Standards for Palustrine Forested Wetland:

| Performance Standard | Documentation |
| --- | --- |
| 3J) Native trees (including both planted and volunteer) in the Palustrine Forested Wetland have a stem density of at least 300 stems/acre at 3 years, 260 stems/acre at 5 years, 230 stem/acre at 7 years, and 200 stems/acre at 10 years following approval of the As-built planting plan. | Monitoring reports documenting tree density approved by the IRT. Stem density for trees within plots will be recorded for years 3, 5, 7, and 10. Trees are defined as those species that have the potential to reach greater than 20 feet in height at maturity. |
| 3K) In the Palustrine Forested Wetland, native woody species have a minimum of 15% cover at 3 years, 35% cover at 5 years, 45% cover at 7 years, and 55% cover at 10 years following approval of the As-built planting plan. | Monitoring reports documenting species occurrence and cover approved by the IRT. Document percent cover in each stratum within the Palustrine Forested Wetland (tree, shrub, herbaceous) in years 3, 5, 7, and 10. |
| 3L) In the Palustrine Forested Wetland, the moisture index is equal to or less than 3.0. | Monitoring reports documenting species occurrence and moisture index approved by the IRT. Document moisture index in monitoring reports for years 3, 5, 7, and 10. |

Performance Standards for Active Floodplain:

| Performance Standard | Documentation |
| --- | --- |
| 3M) In the Active Floodplain, including those AFP areas within the BPA easement, native woody species have a minimum of 15% cover at 3 years, 35% cover at 5 years, 45% cover at 7 years, and 55% cover at 10 years following approval of As-built planting plan. | Monitoring reports documenting species occurrence and cover approved by the IRT. Document percent cover of native woody vegetation for year 3, 5, 7, and 10. |

Objective 4:  Wildlife:

Provide habitat for wildlife in the area by installing habitat features:

| Performance Standard | Documentation |
| --- | --- |
| 4A) Brush piles, cavity trees, nest boxes, and root wads installed according to IRT approved plan. | As-built drawings of habitat features are approved by the IRT. |
| 4B) A minimum of 80 % of the original number of each type of habitat feature will be present at 10 years following approval of As-built plans. | Monitoring reports documenting location of habitat features approved by the IRT. Document location and use of the habitat features at year 10. Document annual maintenance of nest boxes. |

Damage to vegetation in BPA Easement Area:

If activities of the Bonneville Power Administration, or any agent working on its behalf, result in removal of or damage to vegetation within the BPA easement that the Corps and Ecology, in consultation with the Sponsor and the IRT, determine has had a significant adverse impact on the quality of the aquatic functions, native vegetation, soils, or wildlife of the Bank, the Sponsor may request, pursuant to Article III.B. of the Instrument, and the Corps and Ecology, in consultation with the IRT, may approve changes to the performance standards of the Bank.  The expectation is that the Sponsor will be relieved of the obligation to establish, maintain, and monitor the performance standards applicable to BPA-damaged areas within the BPA easement, and approval of any Sponsor request for relief from the applicable performance standards will not be unreasonably withheld.